1. Introduction

1.1 Aortic stenosis and left ventricular haemodynamic burden

Aortic stenosis (AS) is a progressive valvular heart disease characterised by narrowing of the aortic valve and obstruction to left ventricular (LV) outflow. Increasing valvular obstruction imposes a chronic pressure load on the LV, which initially adapts through structural and functional remodelling to maintain cardiac output. Persistent pressure overload can result in LV hypertrophy, impaired relaxation and progressive myocardial dysfunction. Adverse ventricular remodelling may develop before a substantial reduction in left ventricular ejection fraction (LVEF), meaning that preserved systolic function does not necessarily indicate an absence of ventricular consequences.¹

Assessment of AS severity is primarily based on transthoracic echocardiography (TTE), using peak aortic jet velocity, mean transvalvular pressure gradient and aortic valve area (AVA). These measurements remain central to clinical decision-making but are subject to recognised methodological limitations. Doppler-derived velocity depends on alignment between the ultrasound beam and the direction of blood flow, while calculation of AVA depends on measurement of the LV outflow tract. Complex or eccentric flow jets can therefore introduce measurement uncertainty. Discordance between AVA, peak velocity and mean gradient may also occur, particularly when flow conditions or other haemodynamic factors influence the relationship between valve area and transvalvular gradient.¹ Pressure recovery can further contribute to differences between Doppler-derived and invasive pressure measurements.²

The haemodynamic consequences of AS extend beyond measurement of the stenotic valve itself. During ventricular ejection, blood accelerates and converges towards the narrowed valve orifice, while chronic pressure overload can alter LV geometry and filling. These changes may influence the organisation and energetic characteristics of blood flow within the LV. Characterising intraventricular flow may therefore provide a complementary description of the haemodynamic consequences of AS.³

1.2 4D-flow CMR and assessment of left ventricular flow

Four-dimensional flow cardiovascular magnetic resonance (4D-flow CMR) is a time-resolved three-dimensional phase-contrast technique that measures blood velocity in three spatial directions throughout the cardiac cycle. The resulting velocity field provides information about the spatial and temporal organisation of blood flow and permits retrospective interrogation of different regions and planes within the acquired volume.⁴

4D-flow CMR has been investigated as a complementary approach to conventional echocardiographic assessment of AS. Studies have shown that 4D-flow CMR can quantify aortic flow velocity and pressure gradients while providing additional information about the three-dimensional characteristics of stenotic flow.⁵˒⁶ Unlike Doppler echocardiography, 4D-flow CMR is not dependent on alignment with a predefined ultrasound beam and can characterise flow regardless of its spatial orientation. However, 4D-flow measurements remain influenced by acquisition and post-processing factors, including spatial and temporal resolution, velocity encoding and characteristics of the velocity field.⁴ Published work has demonstrated sensitivity of 4D-flow-derived energetic measurements to spatial resolution, while scan-rescan studies have evaluated the reproducibility of LV kinetic energy, viscous energy loss and vorticity measurements.⁷˒⁸ Differences between Doppler echocardiography and 4D-flow CMR may also occur because the two techniques use different acquisition principles and are subject to different methodological limitations.⁵˒⁶˒⁹

The PREFER-CMR registry has investigated the application of 4D-flow CMR in AS. Grafton-Clarke et al. studied 30 patients with AS who underwent TTE and 4D-flow CMR and reported that 4D-flow-derived peak aortic velocity was associated with subsequent aortic valve intervention, whereas the corresponding Doppler-derived measure did not demonstrate the same association.⁹ These findings suggest that 4D-flow CMR may provide complementary haemodynamic information in AS, particularly when conventional measurements are discordant. However, the published analysis focused primarily on aortic haemodynamics and used a different post-processing approach from the MASS-based analysis undertaken in the present study. The present analysis therefore examined LV flow energetics within a subset of the available AS datasets.

An important advantage of 4D-flow CMR is that the same three-directional velocity dataset can be used to investigate LV blood flow across the cardiac cycle. This provides an opportunity to derive multiple complementary measures of intracavitary flow organisation and energetics from a common velocity field.

1.3 Left ventricular flow energetics

Three complementary flow-energetic measures were considered in the present study: kinetic energy, vorticity and energy loss. These measures describe different aspects of intracardiac flow and were therefore considered separately across the cardiac cycle.

LV blood-flow kinetic energy describes the energy associated with the movement of blood within the ventricular cavity and can be derived from the measured velocity field. Normalisation to LV end-diastolic volume (EDV) can account for differences in ventricular size when comparing kinetic energy between individuals. Elhawaz et al. investigated LV blood-flow kinetic energy in 18 patients with severe AS undergoing valve intervention and reported associations between kinetic-energy measures, functional capacity and subsequent LV remodelling.¹⁰ The use of EDV-normalised kinetic energy in the present study allows comparison with previous work using a similar normalisation approach.

Vorticity provides a complementary description of blood-flow organisation by characterising local rotational motion within the velocity field. Because vorticity is derived from spatial velocity gradients, it can capture local flow structures that may not be apparent from scalar measures of velocity or kinetic energy. Rotational flow may occur during both ventricular filling and ejection and may be influenced by ventricular geometry, loading conditions, stroke volume, intracavitary flow patterns and concomitant valvular regurgitation.

Previous studies have demonstrated altered LV vorticity in other cardiovascular and cardiopulmonary conditions. Schäfer et al. reported reduced LV diastolic vorticity in patients with mild-to-moderate chronic obstructive pulmonary disease.¹¹ Demirkiran et al. reported lower mean and peak LV vorticity in patients with chronic myocardial infarction compared with controls, with regional differences according to LV thrombus status.¹² These findings demonstrate that LV vorticity can be quantified using 4D-flow CMR, although the reported abnormalities differ according to disease and cardiac phase. Evidence specifically addressing LV vorticity in AS remains limited.

Energy loss represents the rate at which mechanical energy is dissipated within blood flow. Different 4D-flow approaches have been developed to quantify energy-loss-related parameters, including viscous energy loss and turbulent kinetic energy-related measures.¹³˒¹⁴ These approaches are methodologically distinct, and the present study reports a MASS-derived energy-loss measure without assuming that it represents a specific physical component such as viscous or turbulent energy loss. Evidence describing LV energy loss in AS compared with a control group remains limited.

Together, kinetic energy, vorticity and energy loss provide complementary measures of LV flow behaviour.

Pressure-volume (PV) loop analysis provides another framework for evaluating ventricular energetics, including stroke work and pressure-volume area. These measures describe myocardial mechanical energetics and are conceptually distinct from blood-flow energetic measures derived from 4D-flow CMR.¹⁵ Non-invasive approaches have been developed to estimate PV loops using CMR-derived LV volume curves and brachial blood pressure.¹⁶ Although these measures are physiologically related to blood-flow energetics, they are not interchangeable. The original research proposal included a PV-loop component; however, this component was not completed in the present study. The completed analysis therefore focuses exclusively on blood-flow energetic measures derived from 4D-flow CMR.

1.4 Knowledge gap and study rationale

Despite increasing application of 4D-flow CMR in AS, much of the existing literature has focused on aortic haemodynamics, transvalvular flow and post-stenotic flow patterns.³˒⁵˒⁶˒⁹˒¹³˒¹⁴ Comparatively less is known about how AS influences the organisation and energetic characteristics of blood flow within the LV.

There remains limited evidence examining kinetic energy, vorticity and energy loss together within the LV in participants with AS compared with a control group. The present study therefore undertook an exploratory comparison of these LV flow-energetic parameters between participants with AS and a control group using existing 4D-flow CMR datasets.

The AS and control datasets were derived from the PREFER-CMR registry, as confirmed by the supervisory research team. The AS datasets form part of the cohort reported by Grafton-Clarke et al., who analysed 30 participants with AS from the registry.⁹ Ten control datasets from the same registry were available for the present analysis, although these control datasets have not been reported in a published study. Five AS datasets and five control datasets were included in the present exploratory analysis. The control cohort was not assumed to represent a uniformly healthy volunteer population. AS severity was not used as an eligibility criterion or for selection or stratification in the present analysis.

Given the small analysed sample, the study was exploratory and hypothesis-generating rather than designed to establish diagnostic thresholds, relationships with AS severity or clinical utility. The findings are intended to provide preliminary evidence and methodological insight that may inform larger studies using standardised acquisition, participant selection and flow-analysis methods.

1.5 Aim

To assess differences in left ventricular flow energetics between participants with aortic stenosis and a control group using 4D-flow cardiovascular magnetic resonance.

1.6 Objectives

- To quantify left ventricular flow-energetic parameters, namely EDV-normalised kinetic energy, vorticity and energy loss, using 4D-flow CMR in participants with aortic stenosis and a control group.

- To compare these parameters between the aortic stenosis and control groups across the full cardiac cycle and predefined cardiac phases.

1.7 Hypothesis

It was hypothesised that left ventricular flow energetics would differ between participants with aortic stenosis and a control group.

References — Introduction

- Vahanian A, Beyersdorf F, Praz F, Milojevic M, Baldus S, Bauersachs J, et al. 2021 ESC/EACTS Guidelines for the management of valvular heart disease. Eur Heart J. 2022;43(7):561-632.

- Parameswaran AC, Reisner D, Amanullah A. Discrepancy between gradients derived by cardiac catheterization and by Doppler echocardiography in aortic stenosis: how often does pressure recovery play a role? Echocardiography. 2009;26(9):1000-1005.

- Garcia J, Barker AJ, Markl M. The role of imaging of flow patterns by 4D flow MRI in aortic stenosis. J Cardiovasc Magn Reson. 2019;21:28.

- Dyverfeldt P, Bissell M, Barker AJ, Bolger AF, Carlhäll CJ, Ebbers T, et al. 4D flow cardiovascular magnetic resonance consensus statement. J Cardiovasc Magn Reson. 2015;17:72.

- Adriaans BP, Westenberg JJM, van Cauteren YJM, Gerretsen S, Elbaz MSM, Bekkers SCAM, et al. Clinical assessment of aortic valve stenosis: comparison between 4D flow MRI and transthoracic echocardiography. J Magn Reson Imaging. 2020;51(2):472-480.

- Archer GT, Elhawaz A, Barker N, Fidock B, Rothman A, van der Geest RJ, et al. Validation of four-dimensional flow cardiovascular magnetic resonance for aortic stenosis assessment. Sci Rep. 2020;10:10569.

- Binter C, Gotschy A, Sündermann SH, Frank M, Tanner FC, Lüscher TF, et al. [Use the DOI-imported verified 2016 Binter record from your master reference library.]

- Kamphuis VP, Westenberg JJM, van der Palen RLF, van den Boogaard PJ, van der Geest RJ, de Roos A, et al. Scan-rescan reproducibility of diastolic left ventricular kinetic energy, viscous energy loss and vorticity assessment using 4D flow MRI: analysis in healthy subjects. Int J Cardiovasc Imaging. 2018;34(6):905-920.

- Grafton-Clarke C, Assadi H, Li R, Mehmood Z, Hall R, Matthews G, et al. Four-dimensional flow provides incremental diagnostic value over echocardiography in aortic stenosis. Open Heart. 2025;12(1):e003081.

- Elhawaz A, Archer GT, Zafar H, Fidock B, Barker N, Jones R, et al. Left ventricular blood flow kinetic energy is associated with the six-minute walk test and left ventricular remodelling post valvular intervention in aortic stenosis. Quant Imaging Med Surg. 2021;11(4):1470-1482.

- Schäfer M, Humphries S, Stenmark KR, Kheyfets VO, Buckner JK, Hunter KS, et al. 4D-flow cardiac magnetic resonance-derived vorticity is sensitive marker of left ventricular diastolic dysfunction in patients with mild-to-moderate chronic obstructive pulmonary disease. Eur Heart J Cardiovasc Imaging. 2018;19(4):415-424.

- Demirkiran A, Hassell MECJ, Garg P, Elbaz MSM, Delewi R, Greenwood JP, et al. Left ventricular four-dimensional blood flow distribution, energetics, and vorticity in chronic myocardial infarction patients with/without left ventricular thrombus. Eur J Radiol. 2022;150:110233.

- Dyverfeldt P, Hope MD, Tseng EE, Saloner D. Magnetic resonance measurement of turbulent kinetic energy for the estimation of irreversible pressure loss in aortic stenosis. JACC Cardiovasc Imaging. 2013;6(1):64-71.

- Binter C, Gotschy A, Sündermann SH, Frank M, Tanner FC, Lüscher TF, et al. Turbulent kinetic energy assessed by multipoint 4-dimensional flow magnetic resonance imaging provides additional information relative to echocardiography for the determination of aortic stenosis severity. Circ Cardiovasc Imaging. 2017;10(6):e005486.

- Knaapen P, Germans T, Knuuti J, Paulus WJ, Dijkmans PA, Allaart CP, et al. Myocardial energetics and efficiency: current status of the noninvasive approach. Circulation. 2007;115(7):918-927.

- Seemann F, Arvidsson P, Nordlund D, Kopic S, Carlsson M, Arheden H, et al. Noninvasive quantification of pressure-volume loops from brachial pressure and cardiovascular magnetic resonance. Circ Cardiovasc Imaging. 2019;12(1):e008493.